Software Preparation Before Cluster Analysis

Requirements

1.  Python

Install WinPython 64-bit 3.12.4.1:

https://sourceforge.net/projects/winpython/files/WinPython_3.12/3.12.4.1/Winpython64-3.12.4.1dot.exe/download

After installation, install `wxPython` using `pip`.

2. Insight3

Insight3 is software for single-molecule localization analysis of single-molecule fluorescence images. It was developed by Xiaowei Zhuang's laboratory at Harvard University.

3. MATLAB

Install **MATLAB R2021a** together with all required toolboxes/packages.

===== Cluster diameter and central diffusion coefficients analysis=====

1. Preliminary file preparation

- After drift correction in Insight3, make sure that the drift-corrected “_cor.bin” file is obtained.

- Run the Python diffusion-rate calculation program to generate the “-dis5-avePCABd45nm.bin” file.

2. Preprocessing in Insight3 for Valid-value calculation

Perform the following steps on the “-dis5-avePCABd45nm.bin” file:

- Merge the 1D and 2D data into Category = 0.

- Set the Insight3 parameters: pixel_size = 114 and Ctrl+H diameter = 100 nm.

- Use Ctrl+D to calculate the density and obtain the Valid value. Note: Category merging must be performed before calculating the Valid value.

3. Preparation of regions for analysis in Insight3

It is recommended to create a separate folder for each selected region.

- Select the target region in the “-dis5-avePCABd45nm.bin” file:

  - Record the average diffusion coefficient D(ave) and Valid(ave) of the plasma-membrane region. Avoid clusters and select multiple regions to calculate the average. If the Valid distribution is highly heterogeneous, set the parameters according to the high-density regions.

  - Save the selected region as “c1.bin” (Cluster 1).
If 1D and 2D data need to be analyzed separately, reload the original file before Category merging.

4. Selection of slow-diffusion clusters in Insight3

- Load the “c1.bin” file.

- Set the diffusion-coefficient threshold, usually 0.75 × D(ave).

- Perform filtering in Insight3 using:
c = (zc > threshold ? 5 : c)

- Save the file as “-zc1.5.bin” (example threshold: 1.5).

5. DBSCAN clustering in MATLAB

- Run “dbscan_AllC” on the “-zc1.5.bin” file:

  - Parameters: MinPts = Valid(ave)/4, eps = 0.2.

  - This generates the “-eps0.20_MP50.bin” file (example MinPts = 50).

- Run “cluster_size_dbscan_saveExcel”:

  - Parameter: Filter_n = 1.5 × Valid(ave).

  - Output: “-Filter.xlsx”.

  - Output: “-zc1.5-F.bin”, in which each cluster is assigned to one Frame. Frame = 1 contains all points not assigned to clusters, and Category = 3 contains clusters filtered out by MATLAB.

6. Preparation for diameter calculation

- In the original “_cor.bin” file, select an expanded region around the target region:

  - Extend each side of the target region by 5 pixels.

  - Output file: “part_cor.bin”.

- Run the Python script “step1_find_disp_1-2and2-3”:

  - This generates “part_cor-dis5.bin”.

7. Final diameter calculation in Python

Run the Python script: “Step4_AfterDBSCAN_Diameter”.

- Input files: “-Filter.xlsx” + “part_cor-dis5.bin” + “-zc1.5-F.bin”.

- Key parameters:

  - image_bin_size = 4: diameter range for radial profiling, in pixels. Adjust according to the plotted profile. If the fitted diameter is too large, decrease this value.

  - bin_num_radial = 12: number of radial bins for radial profiling. Adjust according to image_bin_size.

  - fixed_D_fast = 2.2: the D(ave) value.

  - fixed_D_fast_torr: fitting bounds are defined as fixed_D_fast ± fixed_D_fast_torr.

  - diameter_thre = 50: diameter threshold, in nm.

  - diameter_thre_large = 800: large-diameter threshold, in nm.

  - valid_ave = 170: the Valid(ave) value.

- The output Excel file contains the following information for each cluster: xc, yc, zc, count, fitted_zc (slow diffusion coefficient), diameter, zc_bg (fast/background diffusion coefficient), frame, and Valid(ave). This is the final dataset. Data from all cells should be combined into a single Excel file for statistical analysis, and the parameters used for analysis should be recorded.

- Output file: “-zc1.5-F-F2.bin”. In this file, each cluster is assigned to one Frame. Frame = 1 contains all points not assigned to clusters, Category = 3 contains clusters filtered out by MATLAB, and Category = 2 contains clusters filtered out by Python.

Notes

- Keep all parameters consistent throughout the analysis, especially pixel size, disp_thre, and time_interval.

- It is recommended to back up the original files.